# how-to-automate

# How To: Automate Your Joule

Time required: 5–15 minutes per automation Requires: Integration installed and Joule connected — see Getting Started

## Overview

Automations let you control your Joule without touching the app or dashboard. This guide covers the most useful real-world examples.

## Example 1 — Turn the Joule On at a Scheduled Time

Start preheating your water bath before you get home from work.

- Go to Settings → Automations & Scenes → + Create Automation.

- Click Create new automation.

- Under Triggers, click + Add Trigger → Time.

  - Set the time, for example 17:00 (5:00 PM).

- Under Actions, click + Add Action → Call service.

  - Service: switch.turn_on

  - Entity: Sous Vide (your Joule)

- Click Save and name it “Joule — Preheat at 5pm”.

✅ Every day at 5:00 PM, the Joule will start heating the water to 60°C.

## Example 2 — Turn the Joule Off After a Set Duration

Since the current version does not set a cook timer automatically, you can use an automation to turn the Joule off after a number of hours.

- Go to Settings → Automations & Scenes → + Create Automation.

- Under Triggers, click + Add Trigger → State.

  - Entity: Sous Vide (your Joule)

  - To: on

  - For: 02:00:00 (2 hours — adjust to your recipe)

- Under Actions, click + Add Action → Call service.

  - Service: switch.turn_off

  - Entity: Sous Vide (your Joule)

- Click Save and name it “Joule — Auto off after 2 hours”.

✅ The Joule will turn off automatically 2 hours after it is switched on, regardless of how it was started.

## Example 3 — Notify When Water Is Up to Temperature

Get a phone notification when the bath is ready to drop your food in.

- Go to Settings → Automations & Scenes → + Create Automation.

- Under Triggers, click + Add Trigger → Numeric state.

  - Entity: Current Temperature (your Joule)

  - Above: 59.5

- Under Conditions, click + Add Condition → State.

  - Entity: Sous Vide (your Joule)

  - State: on (This prevents the alert from firing if the temperature happens to be warm for another reason.)

- Under Actions, click + Add Action → Send a notification.

  - Message: Your Joule water bath is ready! Drop your food in.

- Click Save and name it “Joule — Ready notification”.

✅ You’ll get a notification as soon as the water reaches 60°C while the Joule is on.

## Example 4 — Voice Control via a Voice Assistant

If you have Google Assistant, Amazon Alexa, or Apple Siri configured in Home Assistant, you can control the Joule by voice once the integration is set up — no extra configuration needed.

| Say this | What happens |
| --- | --- |
| “Turn on the sous vide” | Joule starts cooking |
| “Turn off the sous vide” | Joule stops cooking |
| “What’s the sous vide temperature?” | Reads out the current water temperature |

ℹ️ The exact wake word and phrasing depend on your voice assistant. The entity names can be customised in Settings → Devices & Services to make them easier to say.

## Example 5 — Dashboard Button with a Script

If you want a single dashboard button that starts a complete cook (and another to stop it), use a script.

- Go to Settings → Automations & Scenes → Scripts → + Add Script.

- Give it a name: “Start Joule Cook”.

- Under Actions, add:

  - Call service: switch.turn_on → Sous Vide (your Joule)

- Save the script.

- Repeat to create a “Stop Joule Cook” script with switch.turn_off.

- Add Button cards to your dashboard pointing to these scripts.

✅ You now have clean Start / Stop buttons on your dashboard.

## Tips for Reliable Automations

- Always check if the Joule is available before triggering automations that depend on it. Add a State condition checking that the Sous Vide switch is not unavailable.

- Add a notification for failures. If your Joule goes unavailable during a cook, you’ll want to know. Create an automation that triggers when the Sous Vide entity becomes unavailable and sends you an alert.

- Avoid starting the Joule from both HA and the ChefSteps app at the same time. HA tracks cooking state internally — if you start or stop the Joule from the app, HA won’t reflect that change until you interact with the switch from HA again.

## What’s Next

- Full entity reference →

- Troubleshooting →